# Derivation of the Local Anisotropy Field

Method description for kriging interpolation

14 July 2026

# Derivation of the local anisotropy field

A local anisotropy field (LAF) was derived to guide kriging interpolation in areas where elongated freshwater bodies are expected. These freshwater bodies are assumed to correspond to zones with relatively high resistivity. The analysis was performed separately for each model layer.

The starting point is a gridded observation variable, for example the probability of low resistivity, P(ρ≤5). A proxy for freshwater probability is defined as:

Pfresh=1−P(ρ≤5)

Grid cells with high freshwater probability are selected using a threshold:

Pfresh≥Pfresh,min

These cells are interpreted as observed freshwater cells and are used to estimate the local orientation of freshwater bodies.

# Local PCA on freshwater cells

For each freshwater target cell, neighbouring freshwater cells within a search radius r are selected. If the number of selected cells is smaller than a minimum value nmin, no local anisotropy direction is estimated for that target cell.

A weighted principal component analysis (PCA) is then applied to the coordinates of the selected freshwater cells. The weights are given by the freshwater probability:

wi=Pfresh,i

The weighted mean coordinate is:

x=i​wixii​wi

and the weighted covariance matrix is:

C=1i​wii​wi(xi−x)(xi−x)T

The eigenvalue decomposition of this covariance matrix is:

Cvj=λjvj

with:

λ1≥λ2

The first eigenvector, v1, represents the direction of maximum spatial continuity of the local freshwater body. This direction is used as the local major anisotropy direction.

The corresponding angle is computed as:

θfresh=atan2(v1,y,v1,x)

The angle is measured counter-clockwise from the positive x-axis. Because anisotropy is axial rather than directional, the following equivalence applies:

θ≡θ+180∘

Therefore, all angles are stored modulo 180∘.

The local elongation of the freshwater cells is quantified by the eigenvalue ratio:

Rλ=λ1λ2

Only PCA results with:

Rλ≥Rλ,min

are accepted. This prevents assigning anisotropy directions in areas where the local freshwater-cell geometry is nearly isotropic or poorly defined.

# Filtering against data-support geometry

AEM data are acquired along flightlines, which can introduce a strong sampling geometry. A local PCA may therefore capture the direction of the data support rather than the true geological orientation of a freshwater body.

To reduce this effect, a second PCA is performed within the same search radius, but using all valid data cells instead of only freshwater cells. This gives a local data-support direction:

θdata

and a corresponding data-geometry eigenvalue ratio:

Rλ,data

The axial angle difference between the freshwater PCA direction and the data-support PCA direction is computed as:

Δθ=min|θfresh−θdata| mod 180∘,180∘−|θfresh−θdata| mod 180∘

If the local data-support geometry is itself clearly elongated,

Rλ,data≥Rλ,data,min

and the freshwater direction is too similar to the data-support direction,

Δθ≤Δθmin

the local anisotropy estimate is rejected. This filter suppresses directions that are likely caused by flightline geometry rather than by the geometry of the freshwater body.

# Distance to the transition zone

For each accepted local LAF observation, the distance to the nearest salt-water cell is computed:

dtransition,obs=minjxfresh−xsalt,j

Salt-water cells are defined as:

Pfresh≤Psalt,max

This distance is used as a simple proxy for the distance to the freshwater–saltwater transition zone. Close to such transitions, a relatively short minor anisotropy range is expected. Farther away from the transition, the minor range is allowed to increase.

The PCA step therefore produces sparse, observation-based LAF variables:

θLAF,obs

and:

dtransition,obs

where θLAF,obs is the observed-grid local major-axis angle and dtransition,obs is the observed-grid distance to the nearest transition.

# Interpolation to the full grid

The local PCA analysis only provides LAF estimates at cells where sufficient local support is available. To obtain a full-grid LAF, the sparse observed-grid estimates are interpolated.

Because anisotropy directions are axial, the angles are not interpolated directly. Instead, each angle is transformed using the doubled-angle representation:

u=cos(2θ)

v=sin(2θ)

The components u and v are interpolated separately using inverse distance weighting (IDW):

u(x)=i​wi(x)uii​wi(x)

v(x)=i​wi(x)vii​wi(x)

with IDW weights:

wi(x)=1d(x,xi)p

where p is the IDW power parameter and d(x,xi) is the distance between the prediction cell and an observed LAF cell.

The interpolated major-axis angle is then obtained by transforming back from doubled-angle space:

θLAF(x)=12atan2v(x),u(x)

and is stored modulo 180∘.

The transition-distance proxy is also interpolated using IDW:

dtransition,obs(x)

To account for increasing uncertainty away from cells where the LAF was directly estimated, this interpolated distance is increased by the distance to the nearest observed LAF cell:

dtransition(x)=dtransition,obs(x)+dLAF(x)

where dLAF(x) is the distance from grid cell x to the nearest cell with an accepted local LAF observation.

This approach ensures that the transition-distance proxy increases linearly away from data-supported LAF estimates.

# Conversion to anisotropy ratio

For kriging, the interpolated transition-distance proxy is converted to a local minor range. Given a fixed major range amajor and a minimum minor range aminor,min, the local minor range is defined as:

aminor(x)=clipdtransition(x),aminor,min,amajor

The local anisotropy ratio is then:

rLAF(x)=aminor(x)amajor

Thus:

rLAF=1

corresponds to isotropic interpolation, while smaller values indicate stronger anisotropy. For example, with:

amajor=1000 m

and:

aminor,min=100 m

the anisotropy ratio is constrained to:

0.1≤rLAF≤1.0

The final LAF used for kriging therefore consists of:

θLAF(x)

the interpolated local major-axis angle, and:

rLAF(x)

the local anisotropy ratio.